Data Engineering Challenges for File Storage at Scale

Introduction

When you're scaling a data lake to petabytes, it’s not just about paying for more storage. It’s actually a huge headache for data engineers. Two of the biggest problems I found were the "small file problem" and the absolute mess of managing metadata at that scale.

The Small File Problem

The "small file problem" usually happens because of real-time data. Think about things like IoT sensors or app logs—they’re constantly dumping tiny JSON or CSV files into the lake. While it’s easy to save them that way, it’s a total disaster for queries.

Big data engines like Spark are optimized to read much larger files, usually around 128MB to 1GB. When you try to query a million tiny files, the system has to "open" and "close" every single one, which creates a massive amount of overhead. At companies like Glovo, engineers found that just by smashing these tiny files into bigger ones, they cut their S3 costs by 10x because they weren't paying for millions of individual "GET" requests anymore.

Metadata Management at Scale

The second big challenge is metadata—the "map" that tells the system where all the files are and what’s inside them. In a small setup, it’s no big deal, but when you have billions of files, the metadata itself becomes a bottleneck.

Traditional data lakes are often "write-first, think-later," where anyone can just drop a file. Without a good way to track those files, the lake turns into a "data swamp" where nobody can find anything. Plus, standard object storage doesn't really have "rules," so if a write fails halfway, you end up with half-finished or corrupted data. This is why everyone is moving to Table Formats like Iceberg or Delta Lake. They add a layer that handles the metadata automatically, making sure your queries stay fast even when the data gets massive.